TECHNICAL REFERENCE

Pixboom Spark

UART Control Command Manual

Serial interface settings and control commands for Pixboom Spark.

UART Command Reference

This manual defines the serial port configuration and the supported eight-byte control commands for Pixboom Spark.

1. Serial Settings

| Baud Rate | Data Bits | Stop Bits | Parity |
| --- | --- | --- | --- |
| 115200 | 8 | 2 | None |

Serial shorthand: 115200 8N2

2. Commands

Each command contains 8 hexadecimal bytes. Spaces separate bytes for readability and are not transmitted.

| Function | Command (HEX bytes) | Notes |
| --- | --- | --- |
| Record Start / Stop | 18 33 00 00 00 00 00 00 | Toggle recording start/stop. |
| Exit PRE-REC | 18 33 FF 00 00 00 00 00 | Valid only while the camera is in PRE-REC state. |
| ISO - | 55 54 0D 00 00 00 03 00 | Decrease gain (ISO) by one stop. |
| ISO + | 55 54 0E 00 00 00 03 00 | Increase gain (ISO) by one stop. |
| Shutter - | 55 54 0D 00 00 00 64 00 | Decrease the shutter setting by one stop. |
| Shutter + | 55 54 0E 00 00 00 64 00 | Increase the shutter setting by one stop. |
| Iris - | 55 54 0D 00 00 00 15 00 | Decrease the iris setting by one stop. |
| Iris + | 55 54 0E 00 00 00 15 00 | Increase the iris setting by one stop. |
| White Balance - | 55 54 0D 00 00 00 20 00 | Move white balance one stop down. |
| White Balance + | 55 54 0E 00 00 00 20 00 | Move white balance one stop up. |
| Replay Press | 55 54 21 00 00 00 00 00 | Send when the Replay button is pressed. |
| Replay Release | 55 54 22 00 00 00 00 00 | Send when the Replay button is released. |
| Get Recording Status | 55 66 0A 00 00 00 00 00 | Request the recording-state response. |
| Get All Status | 55 66 0B 00 00 00 00 00 | Request the complete 40-byte status response. |

Response Frames

The camera returns the following frames in response to the two status-query commands.

3.1 Get Recording Status Response

The response contains 3 valid bytes and is zero-padded to a total of 8 bytes.

55 AA 01 00 00 00 00 00

| Offset | Field | Size / Type | HEX | Description |
| --- | --- | --- | --- | --- |
| 0 | SOF1 | 1B / u8 | 55 | First frame-header byte (0x55). |
| 1 | SOF2 | 1B / u8 | AA | Second frame-header byte (0xAA). |
| 2 | RecStatus | 1B / u8 | 01 | Recording-state code; see the value map below. |
| 3-7 | Padding | 5B | 00 00 00 00 00 | Zero padding used to keep the response at 8 bytes. |

RecStatus values: 0x00 INVALID | 0x01 STBY | 0x02 REC | 0x03 PRE_REC | 0x04 PLAYBACK | 0xFF ERROR

3.2 Get All Status Response

Byte order: Little Endian    |    Frame length: 40 bytes

55 AA 01 23 11 00 0F 70
08 E8 03 E8 03 00 00 20
03 64 19 60 FF 1C 00 80
00 00 02 67 01 A5 01 E3
01 2C 01 8D 00 49 01 XX

XX is the runtime CRC-8 byte. The source specification does not define the CRC polynomial, initial value, reflection, or final XOR parameters.

*The complete field layout continues on the next page.

3.2 Get All Status Response - Field Layout

The table below decodes the 40-byte response shown on the preceding page. Offsets are zero-based.

| Offset | Field | Size | Type | Example | HEX | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | SOF1 | 1B | u8 | 0x55 | 55 | First frame-header byte. |
| 1 | SOF2 | 1B | u8 | 0xAA | AA | Second frame-header byte. |
| 2 | RecStatus | 1B | u8 | STBY* | 01 | 00=INVALID, 01=STBY, 02=REC, 03=PRE_REC, 04=PLAYBACK, FF=ERROR. |
| 3 | LEN | 1B | u8 | 35 | 23 | Payload length, used by accessories to determine the remaining frame length. |
| 4 | Version | 1B | u8 | M1/V1 | 11 | High nibble: model; low nibble: protocol version. |
| 5 | Width | 2B | u16 | 3840 | 00 0F | Image width in pixels. |
| 7 | Height | 2B | u16 | 2160 | 70 08 | Image height in pixels. |
| 9 | FPS | 2B | i16 | 1000 fps | E8 03 | Sensor frame rate. |
| 11 | Shutter | 4B | u32 | 1000 us | E8 03 00 00 | Exposure time in microseconds. |
| 15 | ISO | 2B | u16 | 800 | 20 03 | ISO value. |
| 17 | WB_Kelvin | 2B | u16 | 6500 K | 64 19 | White-balance color temperature in kelvin. |
| 19 | WB_CC_x10 | 2B | i16 | -16.0 CC | 60 FF | Raw signed value -160; divide by 10 to obtain -16.0 CC. |
| 21 | Iris_x10 | 2B | u16 | F2.8 | 1C 00 | F-number x10; 0xFFFF means N/A. |
| 23 | MediaUsed | 2B | u16 | 128 GB | 80 00 | Used media capacity in GB. |
| 25 | MediaTotal | 2B | u16 | 512 GB | 00 02 | Total media capacity in GB. |
| 27 | Temp_SOC_x10 | 2B | i16 | 35.9 C | 67 01 | SoC temperature x10 (0.1 C units). |
| 29 | Temp_Sensor_x10 | 2B | i16 | 42.1 C | A5 01 | Sensor temperature x10 (0.1 C units). |
| 31 | Temp_SSD1_x10 | 2B | i16 | 48.3 C | E3 01 | SSD 1 temperature x10 (0.1 C units). |
| 33 | Temp_SSD2_x10 | 2B | i16 | 30.0 C | 2C 01 | SSD 2 temperature x10 (0.1 C units). |
| 35 | Voltage_x10 | 2B | u16 | 14.1 V | 8D 00 | Voltage x10 (0.1 V units). |
| 37 | Power_x10 | 2B | u16 | 32.9 W | 49 01 | Power x10 (0.1 W units). |
| 39 | CRC8 | 1B | u8 | - | XX | CRC-8 checksum used to detect transmission errors. |

* The example frame contains RecStatus 0x01. Per the source value map, this decodes as STBY, despite the source example label REC.